CPE301 – SPRING 2021

# Design Assignment 2A

Student Name: 	    Elmer Mejia

Student #:         	    5003824808

Student Email:  	    mejiae4@unlv.nevada.edu

Primary Github address: https://github.com/cpemejia/design_assignments.git

Directory:		    design_assignments

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmel studio 	d.logic analyzer

- Atmega328pb

- Multifunction shield

Part1:

Part 2

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

;

; DA_2A.asm

;

; Created: 2/22/2021 10:04:43 PM

; Author : ElmerOMejia

;

.INCLUDE "M328pbDEF.INC"

.cseg

.ORG 0x00

LDI R16, LOW(RAMEND)

OUT SPL, R16

LDI R16, HIGH(RAMEND)

OUT SPH, R16

SBI DDRB, 3 ; set PB 3 output

loop:

CBI PORTB, 3 ; turn on portB

LDI R20, 15  ; delay 15x

CALL delay   ; call delay subroutine

SBI PORTB, 3 ; turn off portB

LDI R20, 5   ; delay 5x

CALL delay   ; call delay subroutine

rjmp loop    ; endless loop

delay:       ; delay for a 0.05s delay

LDI R16, 3   ; load immediate 3 to R16

L1:          ; label 1

LDI R17, 105 ; load immediate 105 into R17

L2:

LDI R18, 255 ; load immediate 255, into R18

L3:

NOP          ; NOP for more delay

NOP

NOP

NOP

NOP

NOP

NOP

DEC R18

BRNE L3

DEC R17

BRNE L2

DEC R16

BRNE L1

DEC R20

BRNE delay

ret

*

* DA_2AC.c

*

* Created: 2/23/2021 5:44:39 PM

* Author : ElmerOMejia

*/

#include <avr/io.h>

#define F_CPU 16000000UL // 16MHz

#include <util/delay.h>

int main(void)

{

DDRB |= (1<<PB3);   // set output to pb3

while (1)

{

PORTB &= ~(1<<PB3);  // turn pb3 on

_delay_ms(750);      // delay for 0.75s

PORTB |= (1<<PB3);   // turn off led

_delay_ms(250);      // delay for 0.25s

}

return 1;

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

;

; DA_2A2.asm

;

; Created: 2/23/2021 4:22:58 PM

; Author : ElmerOMejia

;

; Replace with your application code

.INCLUDE "M328pbDEF.INC"

.cseg

.ORG 0x00

LDI R16, HIGH(RAMEND)

OUT SPH, R16

LDI R16, LOW(RAMEND)

OUT SPL, R16

SBI PORTC, 3 ; enable pull up

SBI DDRB, 3 ; set PB3 output

loop:

SBIS PINC, 3 ; skip next if not pressed

RJMP pressed

CBI PORTB, 3 ; turn on portB led

LDI R20, 15  ; delay 15x

CALL delay   ; call delay subroutine

SBI PORTB, 3 ; turn off portB led

LDI R20, 5   ; delay 5x

CALL delay   ; call delay subroutine

RJMP loop    ; endless loop

pressed:

SBI PORTB, 3 ; led pb3 off

SBI DDRB, 2  ; set PB2 as output

CBI PORTB, 2 ; led pb2 on

LDI R20, 40  ; 2s delay

CALL delay

SBI PORTB, 2 ; led pb2 off

SBIS PINC, 3 ; skip next if not pressed

RJMP pressed

RJMP loop

delay:       ; delay for a 0.05s delay

LDI R16, 3   ; load immediate 3 to R16

L1:          ; label 1

LDI R17, 105 ; load immediate 105 into R17

L2:

LDI R18, 255 ; load immediate 255, into R18

L3:

NOP          ; NOP for more delay

NOP

NOP

NOP

NOP

NOP

NOP

DEC R18

BRNE L3

DEC R17

BRNE L2

DEC R16

BRNE L1

DEC R20

BRNE delay

Ret

/*

* DA_2AC.c

*

* Created: 2/23/2021 5:44:39 PM

* Author : ElmerOMejia

*/

#include <avr/io.h>

#define F_CPU 16000000UL

#include <util/delay.h>

int main (void)

{

DDRB |= (1<<PB3);		// set PB3 output

PORTB |= (1<<PB3);      // set PB3 off

//set PORTC.3 for input

DDRC &= (0 << 3);

PORTC |= (1 << 3);

while(1){

PORTB &= ~(1<<PB3); // output PB3 on

_delay_ms(750);     // delay of 0.75s

PORTB |= (1<<PB3);  // output PB3 off

_delay_ms(250);     // delay of 0.25s

if (!(PINC & (1<<PINC3)))

{

PORTB |= (1<<PB3); //PB3 off

PORTB &= ~(1<<2); // set PORTB.2 for output

_delay_ms(2000);  // delay of 2s

PORTB |= (1<<2);  // PB2 off

PORTB &= ~(1<<PB3);		// PB3 output on

}

}

return 1;

}

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

0.75s delay

0.25s delay

Task 2:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://www.youtube.com/channel/UCgodP1yHEZXuYbDRVgoLNzQ/featured

- GITHUB LINK OF THIS DA

https://github.com/cpemejia/design_assignments.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Elmer Mejia