CPE301 – SPRING 2021

# Design Assignment 6

Student Name: 	    Elmer Mejia

Student #:         	    5003824808

Student Email:  	    mejiae4@unlv.nevada.edu

Primary Github address: https://github.com/cpemejia/design_assignments.git

Directory:		    design_assignments

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

-Atmega328pb		-ICM20948

-Microchip Studio	-breadboard

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA_6.c

*

* Created: 5/5/2021 11:07:44 PM

* Author : ElmerOMejia

*/

#define F_CPU 16000000UL			/* Define CPU clock Frequency e.g. here its 8MHz */

#include <avr/io.h>					/* Include AVR std. library file */

#include <util/delay.h>				/* Include delay header file */

#include <inttypes.h>				/* Include integer type header file */

#include <stdlib.h>					/* Include standard library file */

#include <stdio.h>					/* Include standard library file */

#include <avr/interrupt.h>			/* Include avr interrupt header file */

#include <util/twi.h>

#include "i2cmaster.h"

#include "uart.h"					/* Include USART header file */

#include "ICM20948.h"

uint8_t ret;        // return value

uint16_t raw;             // raw sensor value

uint16_t ACC_Data[3], GRYRO_Data[3];          // raw values

void ICM20948_writereg(uint8_t reg, uint8_t val)

{

i2c_start(ICM20948+I2C_WRITE);

i2c_write(reg);  // go to register e.g. 106 user control

i2c_write(val);  // set value e.g. to 0100 0000 FIFO enable

i2c_stop();      // set stop condition = release bus

}

uint8_t ICM20948_readreg(uint8_t reg)

{

i2c_start(ICM20948+I2C_WRITE);  // set device address and write mode

i2c_write(reg);                     // ACCEL_XOUT

i2c_stop();      // set stop condition = release bus

_delay_ms(100);

i2c_start(ICM20948+I2C_READ);    // set device address and read mode

raw = i2c_readNak();                // read one intermediate byte

i2c_stop();

return raw;

}

uint16_t ICM20948_readreg16(uint8_t reg)

{

i2c_start(ICM20948+I2C_WRITE);  // set device address and write mode

i2c_write(reg);                     // ACCEL_XOUT

i2c_start(ICM20948+I2C_READ);    // set device address and read mode

raw = i2c_readNak();                // read one intermediate byte

raw = (raw<<8) | i2c_readNak();     // read last byte

i2c_stop();

return raw;

}

void ICM20948_get_whom_am_I()

{

uint8_t data[1];

ICM20948_writereg(ICM20948_REG_PWR_MGMT_1, 0x80);

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_PWR_MGMT_1, 0x01);

_delay_ms(10);  // Wait for 200 ms.

data[0]=ICM20948_readreg(ICM20948_REG_WHO_AM_I);

if (data[0] != ICM20948_DEVICE_ID)

USART_SendString("Device Not Found\n");

else

USART_SendString("Device Found\n");

}

void Init_ICM20948()

{

i2c_init();     // init I2C interface

USART_SendString("I2C Initialized\n");

_delay_ms(2);  // Wait for 200 ms.

ICM20948_get_whom_am_I();

}

void ICM20948_config(void)

{

ICM20948_writereg(ICM20948_REG_PWR_MGMT_1, 0x80); // reset device

_delay_ms(10); // Wait for 10 ms.

ICM20948_writereg(ICM20948_REG_PWR_MGMT_1, 0x01); // power on

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_BANK_SEL, 0x20);	// Bank 2 select

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_GYRO_SMPLRT_DIV, 0x00); // gyro sample rate

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_ACCEL_SMPLRT_DIV_1,0x00); // accel sample rate

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_GYRO_CONFIG_1, 0x18); // gyro config

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_ACCEL_CONFIG, 0x18); // accel config

_delay_ms(10);

ICM20948_writereg(ICM20948_REG_BANK_SEL, 0x00); // bank 0 select

}

int main(void)

{

char buffer[30], float_[5];

USART_Init(9600);	// usart initialized

USART_SendString("UART Initialized\n");

//	sei();             // Enable global interrupts

_delay_ms(150);

Init_ICM20948();    // Sensor init

ICM20948_config(); // sensor config

_delay_ms(200);     // Wait for 200 ms.

while (1)

{

// Read Accelerometer data

ACC_Data[0]=ICM20948_readreg16(ICM20948_REG_ACCEL_XOUT_H_SH);

ACC_Data[1]=ICM20948_readreg16(ICM20948_REG_ACCEL_YOUT_H_SH);

ACC_Data[2]=ICM20948_readreg16(ICM20948_REG_ACCEL_ZOUT_H_SH);

// Read gyro data

GRYRO_Data[0]=ICM20948_readreg16(ICM20948_REG_GYRO_XOUT_H_SH);

GRYRO_Data[1]=ICM20948_readreg16(ICM20948_REG_GYRO_YOUT_H_SH);

GRYRO_Data[2]=ICM20948_readreg16(ICM20948_REG_GYRO_ZOUT_H_SH);

// divide accelerometer by sensitivity

float acc_x = ACC_Data[0]/ 2048.0;

float acc_y = ACC_Data[1]/ 2048.0;

float acc_z = ACC_Data[2]/ 2048.0;

// divide gyro by sensitivity

float gy_x = GRYRO_Data[0]/ 16.4;

float gy_y = GRYRO_Data[1]/ 16.4;

float gy_z = GRYRO_Data[2]/ 16.4;

// print accelerometer x,y,z axis

dtostrf( acc_x, 3, 2, float_ );

sprintf(buffer," Acc_x = %s\t",float_);

USART_SendString(buffer);

dtostrf( acc_y, 3, 2, float_ );

sprintf(buffer," Acc_y = %s\t",float_);

USART_SendString(buffer);

dtostrf( acc_z, 3, 2, float_ );

sprintf(buffer," Acc_z = %s\n",float_);

USART_SendString(buffer);

// print gyro x,y,z axis

dtostrf( gy_x, 3, 2, float_ );

sprintf(buffer," gy_x = %s\t",float_);

USART_SendString(buffer);

dtostrf( gy_y, 3, 2, float_ );

sprintf(buffer," g_y = %s\t",float_);

USART_SendString(buffer);

dtostrf( gy_z, 3, 2, float_ );

sprintf(buffer," g_z = %s\n",float_);

USART_SendString(buffer);

_delay_ms(1000);

}

}

/*

* uart.h, uart.c, i2cmaster.c, and i2cmaster.h, ICM20948.h

* are not included to save space but were

* taken from cpe301 github.

* They are included in my github.

*/

- SCREENSHOTS OF EACH TASK OUTPUT

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

DA6

- GITHUB LINK OF THIS DA

Design_assign

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Elmer Mejia